# Robot Gesture Control — Command Table

Version: 2025-10-09 • Owner: Team Orangeeeeeeeee
Notes: Sharp turn is one‑shot (edge‑triggered). Smooth turn is continuous and I2C‑safe.

## Quick Params (current defaults)

| Param | Value | Description |
| --- | --- | --- |
| BASE_SPEED | 65 | Forward/Backward baseline speed |
| SMOOTH_TURN_SPEED | 50 | Continuous turn speed (reduced for I2C stability) |
| SHARP_TURN_SPEED | 18 | Sharp turn speed (short pulse on micro:bit) |

## Gesture → Command Mapping

| Gesture (Emoji) | Hand | Command | Behavior | Notes |
| --- | --- | --- | --- | --- |
| 🖐️ Open Palm | Either | CMD:FWD | Move forward (continuous). | Resend every ~0.5 s to keep BLE alive. |
| 👍 Thumb Up | Either | CMD:BACK | Move backward (continuous). | Same resend policy as FWD. |
| ✊ Fist | Either | CMD:STOP | Immediate stop. | Bypasses rate limit when ready. |
| 🖕 One Finger (Middle) | Left | CMD:SMOOTH_LEFT | Continuous pivot/arc turn left. | I2C‑safe soft‑start ramp enabled. |
| 🖕 One Finger (Middle) | Right | CMD:SMOOTH_RIGHT | Continuous pivot/arc turn right. | I2C‑safe soft‑start ramp enabled. |
| ✌️ Peace | Left | CMD:SHARP_LEFT | Short, sharp left pulse. | One‑shot; on‑board pulse + brief brake; requires release. |
| ✌️ Peace | Right | CMD:SHARP_RIGHT | Short, sharp right pulse. | One‑shot; on‑board pulse + brief brake; requires release. |

## BLE / UI Info

UI fields:  BLE status, MAC address, Gesture, Command, Sharp state, FPS.

Sharp is edge‑triggered: hold ✌️ does not repeat; release and re‑pose to trigger again.

Tip: Tune sharpness on-board via SHARP_PULSE_MS and BRAKE_MS; tune responsiveness on PC via stabilizer params.